READ ME FIRST

CTOS Ethernet Data Link for ETH150ISA

CTOS Ethernet Data Link (EDL) version R3.7.0 has added support for the ETH150ISA Ethernet adapter.  CTOS Ethernet Data Link version R3.7.0 package consists of the following installation media:

MEDIA TITLE 	LEVEL	MEDIA TYPE

CTOS Ethernet Data Link (EDL)	R3.6.0	5.25 or 3.5 disk

CTOS Ethernet Data Link (EDLISA)	R3.7.0	3.5 media only

This readme file describes the requirements, installation, and configuration of the EDLISA.  This document (CTOS editor and CTOS Document Designer formats) is also included in the installation media.

Hardware Requirements:

The EDLISA is supported on the following CTOS servers:

SG4000, SG6000, SG7000, SGS4100, SGS6100, SGS7100, SGS4200, SGS3500 and SGS3600.

EDLISA is supported only on ETH150ISA (SMC8416T).  It is not supported on ETH100ISA.  A maximum of one adapter per system is supported.

Software Requirements:

EDLISA requires the following software:

CTOS III 	1.2 or higher

Protocol Manager 	3.1.0001 or higher

CTOS LANceLM 	R2.0.0 or higher

CTOS LANceNW 	R1.1.05 or higher

BTOS OSI TransportLAN	R2.4.1 or higher

BTOS TCPIP	s2.2.4 or higher

Transport Protocols Supported:

EDLISA supports the following protocols:

OSI, TCPIP, NetBIOS, and IPXSPX

Disk and Memory Requirements:

EDLISA requires approximately 364K or RAM and 390Kb of disk space.

Unsupported Features and Configurations:

The following configuration, hardware, software are not supported with EDLISA:

adapters with BNC or AUI connecters (SMC841C,

SMC8416BTETH100ISA)

combination of CTOS Ethernet IOP(s) and ETH150ISA

combination of a Token Ring adapter (SG420C) and ETH150

ISA.

CTOS Clients

BTOS DCA Transport

Configuring the ETH150ISA adapter:

The ETH150ISA uses certain system resources that need to be configured in order to be used with your system.  The adapter is configured through a software program (DOS executible) that is shipped with your ETH150ISA.

What you need to configure the adapter:

DOS bootable disk

ETH150ISA

SMC Superdisk floppy disk that came with the card.  The

disk contains the configuration program EZStart.  Be sure

to read the readme file in this disk.

SMC manual for the card

1.	ETH150ISA requires a free IRQ on the target system.  The first step is to determine what IRQ is available on your target system. CTOS supports IRQ values of 5, 7, 9, 10, and 11.

Boot DOS and run MSD to find out what IRQ is available

2.	The ETH150ISA may be preconfigured with an IRQ of 5 or higher. The IRQ, Base address and IO ports can be changed through software (EZStart) packaged with the card.

3. Install the ETH150ISA card in your system and see if you can boot DOS.

a. If DOS boots then

run the EZStart program to configure the card and also

run the card test program.

note down the configuration parameters:   IRQ, Base

address, ports, etc.

b. If DOS does not boot, then remove any other ISA card from

your system but keep the ETH150 in.

Reboot the system with DOS.

Run EZStart and configure the card to use the IRQ that

you determined in step 1

reinstall all your other ISA cards and reboot DOS

rerun EZStart and test the card

note down the configuration parameters:   IRQ, Base

address, ports, etc.

boot CTOS

c. If the system does not boot in step 'b', then the card has

an IRQ conflict with either the video or disk. In such a

situation, you will need to take the card to another

system where the resources do not conflict.  Then run

EZstart, configure the card, and follow the steps in

'b'.

Installing EDLISA Software Onto a Hard Disk:

Use the following procedure to install the software from distribution media.

1.	At the Executive command line, type Installation Manager;

then press GO.

The Software Operation menu appears.

2.	Follow the instructions that appear on the screen.  Refer to

the CTOS Executive Reference Manual for detailed information

about Installation Manager.

3.	After software installation is completed, make any

configuration file changes as appropriate and reboot the

workstation.

EDLISA Driver Configuration:

The EDLISA driver requires a configuration file.  The driver sets internal software parameters according to the values specified by this file.  However, the EDLISA driver does not change the corresponding parameters on the Ethernet card.  You must set these parameters to match the hardware parameters prior to the EDLISA installation (see configuring the ETH150ISA adapter earlier in this document).

The default EDLISA configuration file name is as follows:

[Sys]<Sys>PktDrvrConfig.sys.

You can override this file name by specifying a name on the installation command form.

In the configuration file, the following parameters are accepted:

:Board ID:

:IRQ:

:IO Base Addr:

:Memory Base:

:Memory Size (K):

The only required parameter is the Board ID parameter.  If you do not specify any of the other parameters or do not indicate values for the parameters, default values are assigned.  Note that the default values may not be suitable for your system. The following is a list of default values used for the ETHISA drivers:

:Board ID:	ETH150

:IRQ:	11

:IO Base Addr:	300H

:Memory Base:	D8000H

:Memory Size (K):	8

IRQ and Memory Size values are in decimal.  The IO Base Addr and Memory Base values are in hexadecimal and may be followed by h or H.  The IO Base Addr is the base IO port (or register) address.  Memory Base is the base address of shared memory.  Memory size is the shared memory size in kilo bytes. Values may be preceded or followed by space andor tab characters. Note that CTOS only supports IRQ values of 5, 7, 9, 10, and 11.

NOTE:  The EZStart program may display base memory as a 4 digit

hexadecimal number.  If so, you will have to add an extra

0 at the end base address e.g. use D8000H for D800

New Command Forms:

The following commands are created during software installation of EDLISA:

Command		Description

Install Packet Ethernet Data Link	Installs EDLISA into memory.

Deinstall Ethernet Data Link  	Deinstalls Ethernet Data Link

from memory.

Display MAC Address  	Displays the MAC Adress of

the card in your system.

Command Form Details:

The Install Packet Ethernet Data Link command installs the EDLISA driver onto the system memory.

Install Packet Ethernet Data Link

[Total Rcv Buffers (def=35)]

[EDL Location (def=not used)]

[Slot Number (def=not used)]

[Cable AUI (A), Twisted (T), Chprnet (C)  (def=A)]

[Link Name (def=EDL)]

[Max.  of LSAPs (def=12)]

[Max.  of Type 2 Connections (def=0)]

[I Frame Acknowledgement? (def=yes)]

[Message File )def=[Sys]<Sys>Pkt>EdlMsg.bin)]

[Protocol Manager Location (def=not used)]

[Config File (def=[Sys]<Sys>PktDrvrConfig.sys)]

Comand Form Parameter 	Description

Total Rcv Buffers (def=35)	Specify the total number of receive buffers EDLISA allocates when installed into memory. Each buffer requires 1 KB of RAM.  You can specify a number from 2 to 128.  Increasing the number of receive buffers may increase performance but leaves less memory for coresiding software.

EDL Location (def=not used) 	This parameter is currently not used.

Slot Number (def=not used)  	This parameter is currently not used.

Cable AUI (A), Twisted (T),	Specify T as the type of

Chprnet (C) (def=A)  	cable the ETH15ISA uses to

connect to the Ethernet

network.

Link Name (def=EDL)	Specify a logical name for this installation of EDLISA.  The link name can be from 1 to 12 characters long.

Max  of LSAPs (def=12)	Specify the maximum number of link service access points (LSAPs) that this installation of EDLISA will support at one time.  The number you specify must be greater than 0.

[Max  of Type 2 Connections	Specify the maximum number of Type 2 (def=0)] 	connections.

[I Frame Acknowledgement? 	This parameter is currently

(def=yes)]	not used.

Message File (def=[Sys]	Specify the path and name of <Sys>Pkt>EdlMsg.bin)	the file containing the data

link's messages.

Protocol Manager Location 	This parameter is currently

(def=not used)	not used.

Configuration File (def= 	Specify the path name of the

[Sys]<Sys>PktDrvrConfig.sys)	Packet Driver Configuration

file.

The Deinstall Ethernet Data Link command deinstalls the EDLISA driver from memory.

Deinstall Ethernet Data Link

[Link Name (def=EDL)]

[Protocol Manager Location (def=not used)]

[Abort Connections? (def=no)]

[Message File (def=[Sys]<Sys>EdlMsg.bin)]

Comand Form Parameter 	Description

Link Name (def=EDL)  	Specify a logical name for this installation of EDLISA.  The link name can be from 1 to 12 characters long.

Protocol Manager Location	This parameter is currently (def=not used)not used.

Abort Connections? (def=no)	Specify yes to deinstall EDLISA even if client applications are still using it;  this may cause abnormal termination for some clients.  Specify no (the default) to deinstall EDLISA only if all clients are finished using it.

Message File (def= 	Specify the path and name [Sys]<Sys>EdlMsg.bin)	of the file containing the

data link's screen messages.

Product Files:

The following files are copied onto your hard disk when EDLISA is installed:

File Name	Description

[Sys]<Sys>PktDrvr.dll  	Dynamiclink library for EDL

ISA

[Sys]<Sys>Pkt>Install8023.run  	Run file for installing EDL>

ISA

[Sys]<Sys>Pkt>Deinstall8023.run  	Run file for deinstalling EDL

ISA

[Sys]<Sys>Pkt>8023.run  	Run file with connectionless

(Type 1) service for EDLISA

[Sys]<Sys>Pkt>EDLmsg.bin  	Message file for EDLISA

[Sys]<Sys>MACAddr.run  	Run file to display MAC

address.

[Sys]<Sys>EDL.E_LANceConfig.hlp  	Online help for the

configuration utility

[Sys]<Sys>EDL.E_LANceConfig.sys  	Location of configuration

information for EDL.

[Sys]<Sys>Pkt>Msg.bin  	Message file for EDLISA

[Sys]<Sys>Pkt>CTOS.dll  	Dynamiclink library for EDL

ISA

[Sys]<Sys>PktDrvrConfig.sys  	Cardadapter configuration

file