DEBUG.DOC                                   June 15, 1987

This disk (Debug Disk #1) contains software  which can be

used  to  debug  the  FlexOS-386  system.  The  debugging

configuration  used  by DRI engineering  consists  of the

following items:

1.  Compaq Deskpro-386 w/ 2 Mb of memory.

2.  IBM-compatible serial adapter (COM 0).

3.  Serial terminal.

4.  A copy of Debug Disk #1 (DOS high-capacity).

5.  OS image file (FLEX386.CMD) and symbols

(FLEX386.SYM) on Debug Disk #1.

SASID386 ("Stand-Alone Symbolic Instruction Debugger") is

written to communicate  directly to COM-0 hardware.   Use

DISKCOPY  (see  FlexOS  User's  Guide)  to make a copy of

Debug  Disk #1.  Do not use the original  Debug  Disk for

debugging.  Use it as a backup.

The FlexOS-386  loadable  image  may be constructed  in 2

forms:  a DEBUG system (which is used with the Debug Disk

and SASID386), and a BOOTABLE system. The main difference

between  the two is the LINK 86 command  options  used to

link  the  systems.  The  Debug  Disk  contains  a  small

single-threaded OS which is loaded automatically when the

Debug Disk is booted on the Compaq.

To create a DEBUG system, the FlexOS image must be linked

to avoid interference with SASID386 or the debug OS.  The

debug  OS resides  from  paragraph  address  0 to 0x2000,

while SASID386  resides in paragraph  addresses 0x7d00 to

0xA000.   This leaves the area between 0x2000  and 0x7d00

paragraphs    to   load   the   FlexOS-386   load   image

(FLEX386.CMD).

There  are two parts to the FlexOS-386  load image - code

and data.   The size of each will vary depending  on what

modules are included  in the system.  Optional OS modules

include linkable  hard disk drivers,  DOS front-end,  and

Graphics  Interface.   To obtain the size of the code and

data in your image  file,  link the file once and LINK 86

will  display  the size of each image  part in hex bytes.

You can then use these numbers to determine where to load

the code and data.  A standard  DRI load address  for the

code section is paragraph 0x200c.  Having selected a code

load  address,  add the size of the code  to it (the code

size  displayed  by LINK  86 is in bytes)  to get  to see

where the code section ends.  Use that paragraph  address

to select the data absolute  load address.   For example,

suppose that the code size is 0x31036 bytes. Convert this

to 0x3104  paragraphs and add it to the code load address

to get 0x5110. This is the 1st legal address for the data

part of the system. For safety we add 0x400 paragraphs to

this number. We now have 0x5510 as the data load address.

There is one more "magic" operation  to perform.   Due to

two data structures in the system that must be aligned on

4K physical addresses (these structures  are the system's

Page Directory  and Page Table),  we must round  the data

load address to a 4K boundary - 20h paragraphs. Therefore

in  our  example  we can  use  0x55e0  as our  data  load

address. At this time you must verify that the end of the

data  section  does  not extend  past  paragraph  address

0x7d00.   In our example that means the data size must be

<= 0x27200 bytes ((0x7d00-0x55e0  paragraphs)  << 4).  If

your  system  is  too big,  you  will  be  unable  to use

SASID386.   To save memory space, certain  drivers may be

dynamically  loaded instead of statically linked into the

FlexOS  image.   This will reduce  the size of FlexOS-386

load  image.  (see FlexOS User's Guide which  talks about

loadable drivers).

The  link  input   file  (see  the  FlexOS   Programmers'

Utilities Guide for more information on LINK 86) would be

as follows:

FLEX386.CMD [code[abs[200c]],

data[abs[55e0],origin[100],max[0]]] =

followed by the names of the objects or librarys that are

used to generate your system.

After  linking  the  system  copy  both  FLEX386.CMD  and

FLEX386.SYM to the root of your copy of the debug disk.

Now place the debug disk in the a:  drive and reboot your

machine.   A batch  file will be automatically  run which

types  this document  and loads  SASID386.   When you see

SASID386's  prompt on your serial terminal,  you may type

'?' for a list of commands supported.

HELPFUL TIPS.

If you are adding  code  to the system,  but not changing

the data size, then place the data as high as possible so

that as the code size increases you can use the same link

input file.

Use  the patch  area  in the system  (symbol  patch)  for

simple conditional  passpoint(s)  routines  while running

SASID386.  See  the  file  SIDTOOLS.A86  for  details  on

writting more complex and useful conditional passpoints.

BUGS.

SASID386   does   not   disassemble   386   instructions.

Conditional  passpoints placed in some real time parts of

the system  can crash the system.   Tracing  in some real

time  parts  of the system  can also  crash  the  system.

SASID's PH command cannot be used until the system is up.