SetGen.DOC

SetGen allows the user to interactively set the display sync

features of the Mindset as well as to set/clear the key bit for

any index in the palette and to set the border color index.

SetGen is an EXE file which should be executed only once to

load it into memory. To actually use SetGen, press the PRTSCN

key while holding down BOTH shift keys.

SetGen accepts two-character commands as follows:

Valid first characters are:

B: Select border color index 0..F

T: Toggle key bit of palette index 0..F

S: Set key bit of palette index 0..F

C: Clear key bit of palette index 0..F

V: Set the Video display sync features to hex value 0..F

<ESC>: Exit

<BS>: Backspace-expect next character to begin a new command

Valid second characters are:

'0'..'F': Hex digit for values 0..15

<ESC>: Exit

<BS>: Backspace-expect next character to begin a new command

SetGen loops on commands until receiving an <ESC> to exit.

SetGen beeps when it is expecting a command character.

SetGen beeeeps on an input error, then waits for the next command.

SetGen does not trap user calls to set screen mode or set palette.

Set screen mode clears the display sync features. Set palette may

change the key bit assignments. If a program does this, it will be

necessary to enter SetGen again through PRTSCN.

SetGen will not operate in programs which have replaced the PRTSCN

interrupt vector (INT 5).