Fixing a Bug in the AT's ROM BIOS

Michael J. Markowitz

Department of Math. Sciences

Loyola Univ. of Chicago

Chicago, IL	60615

(312) 508-3567

After installing an 18MHz crystal in my AT, it scored between 8.4 and 9.4

on Peter Norton's SYSINFO (SI) benchmark, i.e., it clocked in at roughly

8.5 times the speed of an IBM PC.  That's 50% faster than a standard

factory-shipped AT!  But I had one problem--approximately 1 out of every

10 floppy disk accesses would result in a "Disk Read Error" and I'd see

the familiar "Abort, Retry, Ignore?" prompt.  While the retry option

always worked, the interruption of long batch commands began to drive me

crazy.	(I had had absolutely no trouble with the floppy drive prior to

installing the new crystal.)

Checking with a friend at a major software house, I found that this

problem is quite common in speeded-up AT's and that most users have just

learned to live with it.  I considered reinstalling the original crys-

tal, but having become accustomed to "real" speed, could not quite bring

myself to do it.  So suspecting that the problem was in the software--

not the hardware--I began poking around in the ROM BIOS listing in the

Technical Reference Manual and found what I believe is the cause of the

errors.

In the handling of disk I/O requests, the INT 13h service routine goes

into several timing loops while waiting for a response from the NEC

floppy disk controller.  If the NEC fails to respond (via a hardware

interrupt) within a certain time period, the routine "times out" and

returns an error code.	One of these loops (in the WAIT_INT routine)

repeats at most 256K times and would require 25 clock cycles per iter-

ation if executed on an Intel 8088.  On an IBM PC running at 4.77 MHz,

where each clock cycle takes 210 nanoseconds, this would represent a

maximum delay of 2.09 seconds.	A comment in the AT ROM BIOS states that

this loop provides a delay of 2 seconds.  It seems that the folks at IBM

forgot that they were writing code for an 80286 running at 6MHz!

Although incorrectly coded, this timing loop seems barely adequate for

the proper functioning of a standard AT.  Install a faster crystal,

however, and it executes much more quickly, providing a delay which

is often too short for the detection of a response from the (inde-

pendently clocked) floppy disk controller.

It seemed to me that the simplest solution was to write a short routine

which attaches itself permanently to DOS and intercepts all disk I/O

requests through INT 13h (SEE *).  When activated, this routine simulates the

original software interrupt, then checks to see if the BIOS routine has

returned an error code.  If so, and it's a timeout error, the INT 13h (SEE *)

request is repeated, otherwise control returns to the caller.  By the

second request the NEC controller should have issued the expected ack-

nowledgement.

Placing FIXDSK in your autoexec.bat file should completely eliminate

disk access errors.  It has been tested and found to work on three

different AT's, with their original ROMs and crystals ranging in speed

from 16 to 18MHz.  It has not been tested with the BIOS in the new 30MB

AT.

* NOTE: FIXDSK.ASM modified 12/22/85 by C. Washburn to use INT 40H instead of

13H, and to restore contents of AH when retrying. INT 40H change was based

the notion that NOTHING should get in the Fixed Disk INT 13H code path if

at all possible! INT 40H is the relocated address of the BIOS Diskette code,

after the Fixed Disk Controller ROM installs itself, and is only and exclu-

sively for Floppy I/O.