PRNBUF: PARALLEL PRINTER BUFFER Version 1.08

Copyright 1985, 1986 John Navas II, All Rights Reserved

PRNBUF is made available for personal, non-commercial use only.  You

are granted a limited license to use PRNBUF, and to copy it and

distribute it, provided that no fee is charged for such copying and

distribution, and that it is ONLY distributed in its original,

unmodified state.

PRNBUF is unique in that it achieves very high performance for a wide

range of printers with an insignificant amount of processing overhead.

This program works by intercepting calls to the BIOS 17h print request

interrupt routine.  Characters are stored in a 1-64K buffer.  Printing

is attempted when keyboard request (16h), timer tick (1Ch), or printer

ready (0Fh) interrupts occur, minimizing overhead.

Requirements: MS/PC-DOS 2+ and a printer on the first parallel port.

PRNBUF is usually installed in the AUTOEXEC.BAT file.

To install PRNBUF, simply execute it.  If no argument is given, a

64K byte buffer will be allocated; otherwise the buffer size can be

controlled by "/n", where "n" is the size of the buffer in 1,024 byte

increments.  Thus

PRNBUF

installs a 64K byte buffer, whereas

PRNBUF /16

installs a 16K byte buffer.  (It is the user's responsibility to ensure

that sufficient memory is available.)

Once the program is installed, the amount of data in the buffer waiting

to be printed can be determined at any time merely by executing the

program.  If the program is executed with a "/p" (or "/P") option, the

buffer will be purged immediately; if the program is executed with

a "/r" (or "/R") option, the buffer will be purged and the printer will

be reset; and if the program is executed with a "/f" (or "/F") option,

the buffer will be completely flushed to the printer before continuing.

Thus

PRNBUF

reports the data waiting in the buffer, whereas

PRNBUF /p

purges the buffer, and

PRNBUF /r

purges the buffer and resets the printer, and

PRNBUF /f

completely prints the contents of the buffer before continuing.