Date: 10 Mar 83 20:20:43-PST (Thu)

To: net.micro

From: decvax!utzoo!utcsrgv!peterr

Subject: IBM announces new PC, new MS-DOS

As rumoured earlier,  IBM did indeed make a major announcment  on

March  8th.   They announced the IBM Personal Computer XT (a two-

letter  acronym  for extended,  I think;  I guess  only  the  big

computers  get 3LA's),  which comes with 128K std (expandable  to

256K  on the mother board,  using 64K chips),  a 368K  dual-sided

disk, a 10Mb Winchester, and the std IBM Async Board for $7545 in

Canada.   The  new  PC  is a logical redesign  of  the  original,

differentiated  by  its 1/4M mother-board memory capacity and  an

extra  3  expansion slots (though only suitable  for  1/2  length

boards).

I had a look at a PC XT at the local IBM Product Centre and  it's

just about exactly what one would expect.   The system unit looks

like  the  original,  though it's a bit wider to  accomodate  the

extra slots.   The power supply looks improved.  I couldn't get a

good  look  at  the  motherboard as the machine  was  sporting  a

complement of 4 or 5 full length boards.   One surprise was  that

the  10Mb  Winchester  had  a label indicating  it  was  made  by

"MicroScribe", of Colorado, which I've never heard of.

Two  more  new products were in evidence at the  product  centre.

The  IBM PC Colour Display ($1025 Cdn) is a beautiful RGB monitor

styled  like the Monochrome Display.   It is a 13" set which  can

reproduce the IBM's 640x200 resolution graphics mode.   A  manual

for  DOS  2.00 was also available for reference.   It  is  vastly

improved over DOS 1.10,  adding such things as a printer spooler,

user-loadable device drivers, a tree-structured file system (with

mkdir,  rmdir  commands),  enhanced screen support  (e.g.  cursor

control-- possibly  VT100  like  support,   but  I  didn't  check

closely, the manual is very big, rather cramping the std size IBM

binder),  13% disk usage increase (9 sectors per track instead of

8),  a hard disk driver,  and on and on.   Also included is a new

BASIC  for  which documentation wasn't available.   They've  even

added a "more" command,  for 4.1BSD users.   From the OS point of

view, one can have system commands executed from within a program

and there is a distinct notion of a shell.  There is a great deal

that is new,  so I haven't been able to cover it all,  even if  I

could remember all that I read.  The DOS is still very reasonably

priced, at $75 Cdn, making it one of the best deals around.

Announced,  but not in view,  are two expansion units for the PC:

the Model 1,  with a 10Mb drive,  disk adapter, and probably more

slots,  designed  to  be attached to the PC ($5025 Cdn)  and  the

Model 2, which is the same for the PC/XT ($3979 Cdn).

Prices for the basic PC, the colour adapter, and the serial board

have been reduced,  and,  finally,  new versions of the Peachtree

packages, together with their new word processor were announced.

The  press  release claims that all these products are  available

immediately, and the PC XT probably is, but DOS 2.0, for example,

won't be available for separate purchase for at least a month.

Hardly revolutionary announcements, but welcome, particularly DOS

2.00.   It  is interesting to note that the Winchester is not  an

option with the PC/XT; one has to buy it to get the XT, according

to the price list I have.

########################################################################################################