TYPE OZCPR.DOC

OZCPR -- ZCPR (1.0) for Osborne 1.     Page 1

11 Apr 82

Michael M Rubenstein

ABSTRACT.

This  document  describes the  differences  between

OZCPR  and  ZCPR.   It  is  meant  to  be  used  in

conjunction    with   the    ZCPR    documentation.

Instructions  for installation on an Osborne 1  are

included.

OZCPR  requires  Osborne 1 level 1.3.   It has  not

been tested on version 1.2.  It definitely will not

work on Rev A.

DIFFERENCES BETWEEN OZCPR AND ZCPR (1.0).

- The  directory display has been  modified  to

fit on the 52 character display.

- A   disk  reset  is  performed  automatically

before  deleting  files,   saving  files,  or

executing a program.

- OZCPR will run properly on an Osborne 1.

INSTALLATION.

The installation instructions assume that the  user

is familiar with use of DDT.

1.  Using DDT,  check the byte at location 018bh of

setup.com.   It  should be 0c0h.   Change it to

0c9h.    This   change  will  not  affect   the

operation of setup under the standard ccp.   If

this change is not made,  you will not be  able

to use setup under OZCPR.

2.  Copy OZCPR.HEX,  DDT.COM,  and SYSGEN.COM to  a

blank diskette.  This diskette should have been

SYSGENed   with  the  standard  CP/M  operating

system and configured with setup as desired for

the majority of your diskettes.

3.  With this diskette in drive A, type

ddt ozcpr.hex

Then move OZCPR to high memory with

m100,8ff,8000

OZCPR -- ZCPR (1.0) for Osborne 1.     Page 2

Load sysgen with

isysgen.com

r

Clear the default dcb name with

f5d,67,20

Start sysgen with

g100,27f

Sysgen will ask for the source drive.   Respond

A.   Sysgen will then read the operating system

and break to ddt.

Move OZCPR to 4000h with

m8000,87ff,4000

Type g and sysgen will ask for the  destination

drive.   Respond  A.   Sysgen will copy the new

system  to  A.   When sysgen asks  for  another

destination, just respond return to exit.

4.  Press  reset and return to restart  the  system

with the new ccp.

5.  Mount  a  disk on drive B and test the  various

features of OZCPR.   Pay especial attention  to

the directory command.  If there is any problem

because  of  a  change in the  Osborne,  it  is

likely that the most visible effect will be  in

the dir command.  If there is any problem here,

DO NOT CONTINUE.

6.  Use SYSGEN to move the operating system to your

other  diskettes.   Remember  that it  will  be

necessary to use setup to reconfigure any disks

which  do  not  use the  configuration  on  you

initial system.

END OF DOCUMENT

SIG/Access: #D0F0F0FE6604F3E0ACDE0D2D1D5D5

:2005E000A2FE00204CD11C7BFE032002975FD5F52005CD01D01806CD4AD0207CA0