MMDF Dial-up Link Protocol                                    April 1980

1.  Introduction

1.1  Motivation

This document describes the dial-up link level protocol used in the

MMDF electronic mail delivery system.  Currently running on the Unix

operating system, MMDF permits computer systems to exchange messages

over normal telephone connections established with automatic dialing

equipment to regular user ports on time-sharing systems.  The entire

delivery system, including the protocol described in this report, is

implemented as a ordinary user mode program and requires no modifica-

tions to the host operating system.

[Page 1]

MMDF Dial-up Link Protocol                                    April 1980

1.2  Scope

The link protocol described in this report is limited in function to

providing a reasonably robust channel for the transport of 7 bit ASCII

data between communicating processors over ordinary telephone connec-

tions.  Given that the intended application is the efficient exchange

of text messages, only minimal consideration was given to extending

its capabilities, at the link level, to transmission of binary data.

The protocol is packet oriented, and mechanisms for the detection of

data errors and out of sequence packets are provided with recovery

being accomplished through retransmissions.

[Page 2]

MMDF Dial-up Link Protocol                                    April 1980

1.3  Design Considerations

The intent in developing MMDF was to provide a low cost, easily imple-

mentable message delivery system that could be employed by a wide

variety of sites.  Special capabilities in terms of hardware or per-

sonnel should not be necessary for the installation and use of the

system, and the final design should permit relatively simple implemen-

tation on a number of different operating systems.  Given that MMDF is

only a delivery system, as opposed to a complete message reading and

composition environment such as MSG or HERMES, integration with exist-

ing mail systems should be straightforward.

These desires led to the consideration of a number of related issues

in the link protocol design.  In the following sections the major

design points and the rationale behind them will be discussed.  In

virtually all cases, the decision among alternative schemes was made

on the basis of conceptual simplicity and ease of implementation in

restrictive programming environments.

1.3.1  User Emulation

Establishing a connection to another system is done by a host with

automatic dialing equipment in response to either queued mail for

transmission or a clock signal to initiate polling of passive sites

(ie., those without the facilities to originate calls) to request mes-

sages waiting for pickup .  The machine placing the call (the 'active'

or 'master' host) after receiving an indication of successful connec-

tion from the communication hardware, will perform the normal login

procedure on the passive (or 'slave') machine as if it were a regular

user.  When it has successfully identified itself, the active system

will send the appropriate command string to start the protocol program

on the passive host.  This module will announce its readiness by send-

ing a special packet, which when received by the active host, will

start the parameter negotiation sequence described in a later section.

The user emulation approach has the advantage of requiring no special

dial-in ports for the use of the delivery system.  In addition no

modifications to the system login program or command interpreter are

needed.  Particularly in the case of small systems, using existing

dial-in ports can have a significant effect on the cost of using MMDF.

Of course, in a 'network' of MMDF sites, at least one host must have

automatic dialing equipment in order to poll the passive sites and

relay any messages it finds there.

1.3.2  Packet Format

Since the protocol module runs as a normal user program, in the worst

case, input packets must be presented to it as if a user had manually

typed them.  This means that the packets must take the form of an

input line or record and must contain only chracters that a person is

capable of generating from a normal terminal.  Beacuse of this, the

packets processed by the protocol consist of characters from the 7 bit

ASCII set and are terminated by a carriage return.  To the protocol

program on the receiving host, a packet will appear to be an input

[Page 3]

MMDF Dial-up Link Protocol                                    April 1980

line and can be processed with facilities available in the majority of

languages.  Note that this requires no special capabilities in the

protocol module outside of the ability to turn off character echo to

the terminal.  This 'echo suppression' is necessary to prevent packets

sent by a host from being automatically sent back to the transmitter

and being interpreted as protocol output from the other machine.  It

is not necessary for the program to be able to disable terminal echo.

If disabling echo is difficult or impossible to do from inside a user

program, but the system has a command to perform this function, the

active host can send the appropriate string before starting the proto-

col.

1.3.3  Transparent Text Transmission

The issue of transparent transmission arises from the imposed con-

straint of allowing the link protocol to be implemented as a user pro-

cess on virtually any operating system.  Since the intent of the sys-

tem is to allow the exchange of text messages which may contain any of

the 7 bit ASCII codes, for highest efficency, the input character set

should not be restricted by the host operating system.  Unfortunately,

most systems ignore certain input characters (such as null's used for

padding) or interpret them for special functions such as line editing

or program termination.  Recognizing that in some systems it may not

be possible to disable these actions, the link protocol incorporates a

mechanism for translating certain subsets of the ASCII characters into

unique sequences of legal (ie. uninterpreted) input characters in

packets sent to be sent to the receiving host.

This problem is not restricted to the actions of the recipient

machine; the communication channel itself might be incapable of pass-

ing certain characters without special effects.  As an example, in the

original experiments with the MMDF system, one of the machines used is

the Unix system at the Rand Corporation in Santa Monica, California

(Rand-Unix).  Access to that system was accomplished via the Arpanet

by dialing a local terminal access machine, either an ANTS or a TIP.

Both of these systems have 'escape' characters which preceed commands

meant for the ANTS or TIP itself rather than the host to which the

user is connected.  If these characters ('^' for ANTS, '@' for TIP's)

were present in a packet sent via one of these systems, the remainder

of the packet would be interpreted as a command by the communication

system with unpredictable results.

Because of these potential character set problems, the link protocol

incorporates a scheme for determining, on a per connection path basis,

which characters have to be encoded during transmission.  As an exam-

ple, suppose host A contacts host B to exchange some messages.  System

B is aware of which characters it is unable to receive without inter-

ception by its operating system, and it sends a packet informing A of

their identity.  On the other hand, A is aware of which characters it

is unable to transmit, due to limitations in its executive and/or the

properties of the communication channel used to reach B.  Combining

the information about itself with the data sent by B, host A forms the

legal character set for transmission from itself to B and thereby

determines which characters have to be encoded.  This sequence only

characterizes the transmission of packets from A to B; a similar

[Page 4]

MMDF Dial-up Link Protocol                                    April 1980

operation is performed for the opposite path.

1.3.4  Packet Length

For the same reason that the transparent text issue was important,

namely the varying capabilities among operating systems, consideration

must also be given to determining the maximum packet length that can

be sent over any connection.  This length will have a marked effect on

throughput, since as will be discussed in the next section, the proto-

col is designed to be primarily half-duplex.  Using short packets will

necessitate more acknowledgements per block of text transmitted, and

will also raise the ratio of overhead characters (such as checksums)

to text characters.

As before, negotiation between the communicating hosts is used to

determine the maximum packet length.  When the protocol module on Host

B is started by Host A, B will send the length of the longest input

line that it can handle along with the information about its tran-

sparent input character set.  When A receives this data, it will

determine the longest packet that can be sent from itself to B by tak-

ing the minimum of (1) the number of characters in the longest line A

can transmit, and (2) the maximum nuber of characters per line that B

can receive.

To get the maximum possible efficiency, different maximum packets

lengths are allowed for each of the two transmission paths in a con-

nection (ie. A to B and B to A).

1.3.5  Half/Full Duplex Operation

Another important consideration in the design of the protocol was the

support of full duplex communications.  If the channel used is capable

of full duplex operation, then for maximum throughput the protocol

used should be designed to allow a host to be sending the acknowledge-

ment for one packet while receiving the next one in the sequence.

Implementing such a scheme as a user process on less flexible operat-

ing systems can be quite problematic.  To be able to program such a

protocol, the environment must provide either (1) a non-blocking or

asynchronous I/O capability to the terminal or (2) a multi-process

facility accessable by the user with a reasonable inter-process com-

munication scheme.  If a system lacks both of these abilities, it is

easy to see the problems if one considers what would happen in host B

should B be sending a packet to host A, and be blocked in the write

call, while A is sending a packet to B.  If host B does not provide

reasonable read-ahead on its terminal inputs, then if A starts sending

another packet before B could read the previous one (because it was

hung in the write call), then the second packet would be lost.  It is

also the case with some systems that the machine is simply incapable

of receiving data on a tty line while it is also transmitting on that

line.

Because of the desire to have the protocol implementable on the least

hospitable systems, the decision was made to make the protocol half-

duplex regardless of the capabilities of the communication channel.

[Page 5]

MMDF Dial-up Link Protocol                                    April 1980

This considerably simplifies the design and programming but suffers

from reduced throughput.

1.3.6  Preserving User Packet Size

As discussed previously, the physical packet size used during a con-

nection is negotiated at the start of communication, and is not likely

to be the same for different pairs of communicating hosts.  The appli-

cation program, such as the MMDF mail delivery protocol module, should

not be required to have any knowledge of the size of the packets being

sent between two systems.

The user may wish to structure the data that he is sending into logi-

cal records, and have the boundaries of these records indicated to the

application program on the receiving machine.  To permit this, the

link protocol has a facility for marking a physical packet as contain-

ing the end of a logical record.

Logical records can be any length, and if necessary, they will be

fragmented into multiple physical packets, the last of which will con-

tain the end-of-segment mark.  The fragments are transmitted and

delivered to the receiving in order.  The application program is in-

formed when the packet containing the mark is received.

[Page 6]

MMDF Dial-up Link Protocol                                    April 1980

1.4  Environment

The link protocol described in this report, and the module which im-

plements it, are designed to accept data from the mail delivery pro-

cess on one system, transmit it across the communication path, and

pass it to the corresponding delivery process on the recipient host.

The link module takes logical records from the application progra as

input, constructs the appropriate physical packets, and transmits the

result.  The companion program on the other end of the connection

receives the packets from the terminal line, disassembles them, and

forwards the data (and an indication of the ends of the logical

records) to its attached delivery process after returning the ap-

propriate acknowledgement to the transmitting machine.  The hosts'

operating systems are used for I/O because of the requirement that the

link protocol be implementable as a normal user program.

The diagram below illustrates the relationships just described between

the various components of the communication system on one end of a

connection.

+-------------------------------+

|  MMDF Mail Delivery Protocol  |

+-------------------------------+

|

|  <---- User Logical Records

|

+-------------------------------+

|     Dial-up Link Protocol     |

+-------------------------------+

|

|  <---- Physical Packets

|

+-------------------------------+

|     Host Operating System     |

|     Terminal I/O Facility     |

+-------------------------------+

|

|

|

|-------------------... Terminal Line to

Other Host

Figure 1.4.0 - Link Protocol Environment

The above diagram is not intended to suggest that the link and mail

delivery protocol modules are to be implemented as separate processes.

As discussed earlier, the protocol described in this report is

designed to be programmable in uni-process environments.  In fact, the

current Unix version is coded as a collection of subroutines which are

bound into the delivery process during compilation.

The user's logical records passed to (and received from) the link pro-

tocol module contain data relevant to the overall function of the

delivery system, and is not interpreted by the lower level (ie. the

[Page 7]

MMDF Dial-up Link Protocol                                    April 1980

link protocol).  The data given to the upper level by the link module

is unchanged in content from that presented to the equivalent module

on the other end of the connection.

[Page 8]

MMDF Dial-up Link Protocol                                    April 1980

2.  Definition

2.1  Packet Format

Packets, as received by a host, consist of a sequence of 8 bit charac-

ters, terminated by a carriage return.  Only the lower 7 bits of each

character are significant, with parity set as appropriate for the

receiving host.  The character set used is ASCII.

The maximum number of characters in a packet travelling in a given

direction is determined by negotiation between the transmitting and

receiving hosts, and may not be greater than 255 nor less than 40 (the

length of an XPATH or RPATH packet) exclusive of the carriage return.

All packets have a common header, followed by an optional text field

whose format is dependent on the packet type.  The characters are

transmitted in the order indicated by the byte numbers as shown in

Figure 2.1.0.

+-----+-----+-----+-----+-----+-----+-----+-----+---//---+-----+-----+

|       Checksum        | Type|Flags|       Type-Specific Text       |

+-----+-----+-----+-----+-----+-----+-----+-----+---//---+-----+-----+

1     2     3     4     5     6     7     8             N-1    N

Figure 2.1.0 - General Packet Format

Checksum - the unsigned 16 bit sum of characters 5 to N expressed as a

hexadecimal number, with digits from the set

(0,1,2,...8,9,A,B,...E,F).  Only the lower 7 bits of each charac-

ter are used in the calculation, and the most significant digit of

the result is placed in character position 1.

Type - a single character code which indicates the type of the packet.

The defined types and their codes are:

Code Char     Description

0         DATA - transmit user data

1         DATAACK - acknowledge DATA packet

2         XPATH - characterize transmit path

3         XPATHACK - acknowledge XPATH packet

4         RPATH - characterize receive path

5         RPATHACK - acknowledge RPATH packet

6         ESCAPE - send receive escape character

7         ESCAPEACK - acknowledge ESCAPE packet

8         QUIT - cause remote protocol to exit

9         QUITACK - acknowledge QUIT packet

Flags - a single hexadecimal digit indicating the state of 4 bits.

This field is discussed in Section 2.1.0.

[Page 9]

MMDF Dial-up Link Protocol                                    April 1980

Type-Specific Text - this field contains data whose format and seman-

tics are dependent on the specific packet type, as indicated by

byte  5.  Not all packets utilize the text field however, but for

those that do, the contents of this field are described in the

individual packet descriptions in Sections 2.1.1 to Section

2.1.10.

2.1.0  Flags Field

Byte  6, the Flags character, is a single hexadecimal digit which

gives the state of the 4 bits shown in Figure 2.1.0.0.

+-----+-----+-----+-----+

|Orig.| Eof | Seq. Num. |

+-----+-----+-----+-----+

3     2     1     0

Figure 2.1.0.0 - Flag Field Format

Origin - this bit is set if the packet is being sent by the host that

initiated the phone call.  It is used by a system to assure that

it does not process received packets that it had transmitted due

to the remote host having not disabled character echo.

Eof - this bit is used as the stream marker to mark the end of a logi-

cal data record.  This facility is used to give the program using

the protocol a means of using logical packets of any size, regard-

less of the physical packet size in use in the link protocol.

Sequence number - these bits form a 2 bit binary number which incre-

ments with each successfully transmitted non-acknowledgement pack-

et.  Their function is to allow the detection of missing or out of

order packets.  Acknowledgement packets (DATAACK, XPATHACK,

RPATHACK, ESCAPEACK, and QUITACK) have the sequence number of the

packet they are acknowledging.

2.1.1  DATA Packet

+-----+-----+-----+-----+-----+-----+-----+-----+---//---+-----+-----+

|       Checksum        |  0  |Flags|        Encoded User Data       |

+-----+-----+-----+-----+-----+-----+-----+-----+---//---+-----+-----+

1     2     3     4     5     6     7     8             N-1    N

Figure 2.1.1.0 - DATA Packet Format

The DATA packet is transmitted by a host to send user data (as opposed

to protocol control information) over the connection.  The data to be

sent is encoded as described in Section 2.3 and placed in the text

field of the packet.  A packet must contain the encodings for a in-

tegral number of original, unencoded characters, ie. character encod-

[Page 10]

MMDF Dial-up Link Protocol                                    April 1980

ing strings may not be split across packet boundaries.

If a DATA packet is successfully received by the other host, a DATAACK

packet will be returned.

2.1.2  DATAACK Packet

+-----+-----+-----+-----+-----+-----+

|       Checksum        |  1  |Flags|

+-----+-----+-----+-----+-----+-----+

1     2     3     4     5     6

Figure 2.1.2.0 - DATAACK Packet Format

The DATAACK packet is transmitted by a host to acknowledge the suc-

cessful receipt of a DATA packet.  The sequence number in the DATAACK

is the same as the packet being acknowledged.

2.1.3  XPATH Packet

+-----+-----+-----+-----+-----+-----+-----+-----+-----+---//---+-----+

|       Checksum        |  2  |Flags|Max. Packet| Legal Input Chars. |

+-----+-----+-----+-----+-----+-----+-----+-----+-----+---//---+-----+

1     2     3     4     5     6     7     8     9              40

Figure 2.1.3.0 - XPATH Packet Format

The XPATH packet is transmitted by a host to indicate to the remote

system (1) the maximum line length that it can transmit, and (2) which

characters in the complete 7 bit ASCII set it is incapable of

transmitting.  A correctly received XPATH packet is acknowledged with

an XPATHACK packet.

Bytes 7 and 8 of the packet contain a 2 digit hexadecimal number which

is the maximum number of characters in a packet that can be transmit-

ted by the host, minus the carraige return.  Byte 7 is the most signi-

ficant digit.

Bytes 9 through 40 contain 32 hexadecimal digits which are an encoding

of the illegal output character set for the host.  The 128 possible

codes are represented as a bit vector, where the index of a bit is the

ASCII value of the character.  A given bit in the vector is set if the

corresponding character cannot be transmitted without side effects by

the host.

This vector is broken up into 32 four bit pieces, each of which is

represented as a hex digit and placed in this field in the packet.

Byte 9 corresponds to bits (and ASCII values) 0 to 3, byte 10 to bits

4 to 7, and so forth.  The hex conversion of each of the 4 bit groups

considers the most significant bit to be the one with the highest

index in the original vector, as shown below.

[Page 11]

MMDF Dial-up Link Protocol                                    April 1980

ASCII values ---> 11        8  7        4  3        0

(indicies)        0  1  1  0  1  0  0  1  1  0  1  1  <--- bit vector

----------  ----------  ----------

|           |           |

|           |           |

hex codes --->  6           9           B

Figure 2.1.3.1 - Example of Legal Input Character Set Encoding

In the example, only the characters with the values 0, 1, 3, 4, 7, 9,

and 10, out of the first 12 possible codes, can be received by the

host.  The first three hex digits corresponding to this subset would

be B, 9, and 6, and would be loaded into bytes 9, 10, and 11 respec-

tively.

The illegal character information in the XPATH packet is used by the

remote host (along with knowledge about its own environment) to deter-

mine the escape character that the system sending the XPATH should use

for encoding user data to be transmitted.  The sender is informed of

the identity of the character in a subsequent ESCAPE packet.

2.1.4  XPATHACK Packet

+-----+-----+-----+-----+-----+-----+

|       Checksum        |  3  |Flags|

+-----+-----+-----+-----+-----+-----+

1     2     3     4     5     6

Figure 2.1.4.0 - XPATHACK Packet Format

The XPATHACK packet is transmitted by a host to acknowledge the suc-

cessful receipt of a XPATH packet.  The sequence number in the

XPATHACK is the same as the packet being acknowledged.

2.1.5  RPATH Packet

+-----+-----+-----+-----+-----+-----+-----+-----+-----+---//---+-----+

|       Checksum        |  4  |Flags|Max. Packet| Legal Input Chars. |

+-----+-----+-----+-----+-----+-----+-----+-----+-----+---//---+-----+

1     2     3     4     5     6     7     8     9              40

Figure 2.1.5.0 - RPATH Packet Format

The RPATH packet is transmitted by a host to indicate to the remote

system (1) the maximum line length that it can receive, and (2) which

characters in the complete 7 bit ASCII set it is incapable of receiv-

ing without side effects, such as line or character delete.  A

correctly received RPATH packet is acknowledged with an RPATHACK pack-

et.

Bytes 7 and 8 of the packet contain a 2 digit hexadecimal number which

[Page 12]

MMDF Dial-up Link Protocol                                    April 1980

is the maximum number of characters in a packet that can be received

by the host, minus the carriage return.  Byte 7 is the most signifi-

cant digit.

Bytes 9 through 40 contain 32 hexadecimal digits which are an encoding

of the illegal input character set for the host.  The format of this

field is identical to that used in the RPATH packet described in sec-

tion 2.1.3.  Bits are set in the vector if the corresponding character

cannot be received at all, or without side effects.

The illegal character information in the RPATH packet is used by the

remote host (along with knowledge about its own environment) to deter-

mine which characters it has to encode before transmission to ensure

successful delivery.  Similarly, the maximum packet length specifica-

tion gives an upper bound on the length of packets that can be re-

ceived.  The maximum packet length that a host can transmit on a con-

nection is the minimum of (1) the maximum length that can be received

by the remote host and (2) the maximum length that the local host can

transmit.

The successful receipt of an RPATH packet is acknowledged by an

RPATHACK packet.

2.1.6  RPATHACK Packet

+-----+-----+-----+-----+-----+-----+

|       Checksum        |  5  |Flags|

+-----+-----+-----+-----+-----+-----+

1     2     3     4     5     6

Figure 2.1.6.0 - RPATHACK Packet Format

The RPATHACK packet is transmitted by a host to acknowledge the suc-

cessful receipt of a RPATH packet.  The sequence number in the

RPATHACK is the same as the packet being acknowledged.

2.1.7  ESCAPE Packet

+-----+-----+-----+-----+-----+-----+-----+-----+

|       Checksum        |  6  |Flags| Esc. Char |

+-----+-----+-----+-----+-----+-----+-----+-----+

1     2     3     4     5     6     7     8

Figure 2.1.7.0 - ESCAPE Packet Format

The ESCAPE packet is sent to a host to inform it which character

should used as the 'escape' in the encoding of user data sent in the

text field of DATA packets.  ESCAPE's are sent after a exchange of

XPATH and RPATH packets, which allow the hosts to detmine the legal

character sets for the two transmission paths of a connection.  The

escape character is chosen from 7 bit ASCII such that (1) it can be

transmitted by the remote host, (2) it can be received by the local

[Page 13]

MMDF Dial-up Link Protocol                                    April 1980

host, and (3) it occurs as infrequently as possible in the text to be

transmitted as user data.  The third requirement is important so that

character encodings can be minimized, and the throughput of the con-

nection as high as possible.

ESCAPE packets are sent by each of the hosts in a connection and there

is no requirement that the escape characters be the same for both

transmission paths.

Bytes 7 and 8 of the ESCAPE packet contain a two digit hexadecimal

number giving the ASCII code of the selected escape character.  Byte 7

is the most significant.

2.1.8  ESCAPEACK Packet

+-----+-----+-----+-----+-----+-----+

|       Checksum        |  7  |Flags|

+-----+-----+-----+-----+-----+-----+

1     2     3     4     5     6

Figure 2.1.8.0 - ESCAPEACK Packet Format

The ESCAPEACK packet is transmitted by a host to acknowledge the suc-

cessful receipt of a ESCAPE packet.  The sequence number in the

ESCAPEACK is the same as the packet being acknowledged.

2.1.9  QUIT Packet

+-----+-----+-----+-----+-----+-----+

|       Checksum        |  8  |Flags|

+-----+-----+-----+-----+-----+-----+

1     2     3     4     5     6

Figure 2.1.9.0 - QUIT Packet Format

The QUIT packet is sent to inform the remote host that the local pro-

tocol module is exitting.  When received by the remote system, The

local host must wait to receive a matching QUIT before exitting.

2.1.10  QUITACK Packet

+-----+-----+-----+-----+-----+-----+

|       Checksum        |  9  |Flags|

+-----+-----+-----+-----+-----+-----+

1     2     3     4     5     6

Figure 2.1.10.0 - QUITACK Packet Format

The QUITACK packet is transmitted by a host to acknowledge the suc-

cessful receipt of a QUIT packet.  The sequence number in the QUITACK

[Page 14]

MMDF Dial-up Link Protocol                                    April 1980

is the same as the packet being acknowledged.

[Page 15]

MMDF Dial-up Link Protocol                                    April 1980

2.2  Acknowledgement Policy

Acknowledgements (ACK's) are transmitted in response to all correctly

received packets except acknowledgements themselves.  Each data and

information packet type has a specific acknowldgement packet associat-

ed with it to aid in the detection of protocol errors.

There is no provision for explicit negative acknowledgement.  Instead,

if no ACK is received by a host within a reasonable time after sending

a packet which requires one, the packet is retransmitted.  If no suit-

able response has been received after several retransmissions, the

connection can be considered dead.  The timeout period and the number

of retries is a fairly important parameter for tuning particularly

when communicating between heavily loaded systems.  Due to the im-

plemenataion as a regular user process, heavy system use introduces a

good deal of variance in the response times to incoming packets.

Short timeouts will necessitate unnecessary retransmissions and may

cause dropped connections because of tardy replies from a host.

To avoid ambiguities, ACK's contain the sequence number of the packet

being acknowledged.  A packet is considered acknowledged only when an

ACK is received by the transmitting host with a sequence number which

matches the one in the original packet.  All other ACK's are ignored.

In addition, out of sequence packets of all types are acknowledged,

but ignored.  This takes care of the case in which a packet was re-

ceived correctly, but its acknowledgment was lost, causing a re-

transmission.

[Page 16]

MMDF Dial-up Link Protocol                                    April 1980

2.3  Character Encoding

The encoding method used for non-transparent characters substitutes a

unique sequence of legal characters, beginning with a predetermined

lead-in or escape character, to represent those codes which cannot be

transmitted normally.  For example, if it is impossible to receive !,

?, A, g, and <ctrl>Z, and the escape is '~' (tilde), the encoding

might be as shown in Figure 2.3.0.

Character    Encoding

---------------------

!          ~21

?          ~3F

A          ~41

g          ~67

<ctrl>Z       ~1A

Figure 2.3.0 - Character Encoding Example

The two characters following the escape are the ASCII value of the

original character pressed as two hexadecimal digits.

In this arrangement, it is also necessary to provide an encoding for

the escape character itself.  The technique used in the protocol is to

simply send the escape code twice in succession for each occurance in

the original text.  Therefore, following the preceeding example, if

'~' was present in the original text it would be transmitted as '~~'.

While the characters that have to be encoded will in general vary from

connection to connection, the escape sequence encodings for all of the

ASCII characters are predetermined.  However, an appropriate escape

character must be chosen for each connection that occurs infrequently

in normal message text, so that the escape itself will rarely have to

be encoded.

Experience with an earlier version of this protocol has indicated that

by far the largest number of characters that required encoding to

prevent interpretation by the receiving host operating system were the

ASCII control characters, ie. those with values between 0 and 37

(octal), along with DELETE (177 octal).  Recognizing that a few of

these codes are quite common in normal text files and messages, namely

tab <tab>, line feed <lf>, form feed <ff>, and carriage return <cr>, a

special encoding for these control characters has been specified.

This encoding uses 2 characters instead of 3 as required by the scheme

shown in Figure 2.3.0, and is intended to maximize the protocol's use

of the available channel bandwidth.

The special encodings for these characters is shown in Figure 2.3.1,

which again assumes that '~' is the escape character.

[Page 17]

MMDF Dial-up Link Protocol                                    April 1980

Character    Encoding

---------------------

<tab>        ~I

<lf>         ~J

<ff>         ~L

<cr>         ~M

Figure 2.3.1 - Character Encoding Example

It should be noted that the transmission of these special sequences is

optional; a host is free to encode all the control characters using

the 3 character hex method.  However, a host is required to be capable

of receiving and correctly interpreting the 2 character codes.

[Page 18]

MMDF Dial-up Link Protocol                                    April 1980

2.4  Protocol Operation

The operation of the protocol is illustrated in Figure 2.4.0.  Host A

has placed a phone call to Host B, logged itself in as a user, and has

issued the command necessary to start the slave protocol module on B.

Using earlier terminology, Host A is the master and Host B is the

slave.

Event     Host A                 Host B

---------------------------------------

|                    <---- XPATH

1    |

|  XPATHACK ---->

|                    <---- RPATH

2    |

|  RPATHACK ---->

|  XPATH ---->

3    |

|                 <---- XPATHACK

|  RPATH ---->

4    |

|                 <---- RPATHACK

|                   <---- ESCAPE

5    |

|  ESCAPEACK ---->

|  ESCAPE ---->

6    |

|                <---- ESCAPEACK

- - - - - - - - - - - - - - - - - - - -

|  DATA ---->

7    |

|                  <---- DATAACK

|                     <---- DATA

8    |

|  DATAACK ---->

- - - - - - - - - - - - - - - - - - - -

|  QUIT ---->

9    |

|                  <---- QUITACK

---------------------------------------

Figure 2.4.0 - Example Protocol Sequence

Event 1 occurs immediately after the protocol module on Host B has

been started.  B sends an XPATH packet giving the maximum length of

the packets that it can transmit and the characters that it is incapa-

ble of sending.  Host A responds with an XPATHACK when it has been

[Page 19]

MMDF Dial-up Link Protocol                                    April 1980

correctly received.

In Event 2, Host B sends an RPATH packet with the maximum packet

length that it can receive ad an encoding of the characters that it

cannot receive without adverse effects.  Host A responds with an

RPATHACK.

Next, Host A sends its XPATH and RPATH packets and receives their ack-

nowldgements.

Once all the PATH packets have been exchanged, the hosts have the

information necessary to determine which characters will need encoding

for transmission in each direction, and the maximum length packets

that can be sent.  Each host then chooses the escape character that

the opposite system will use in encoding the text in DATA packets.

The escape characters are then exchanged as shown in Events 5 and 6.

Events 7 and 8 illustrate canonical data transfer transactions.  The

order of host transmissions of DATA packets is not governed by the

link protocol, provided of course, that each packet is acknowledged

before the another one is sent by either system.

The standard way of shutting down the protocol is shown in Event 9.

The host that determines that the connection should be dropped sends a

QUIT packet, which is acknowledged by the receiver.  Once this ex-

change has been made, the connection is considered closed and both

protocol modules provide the appropriate indications to the driving

programs.

[Page 20]

#################################################################################################################