ViewRetargeter2.2.1 Release 6/28/93

===================================

Note: release note for 2.2.1 comes in four sections:  enhancements,

bug fixes, application notes, and Target libraries.

Enhancements:

1. Retargeter now works with the new Actel ALS2.2 design_kits.

2. The I/O macroes in Actel design, including bbdlhs, bibuf, clkbibuf,

clkbuf, ibdl, inbuf, ir, obdlhs, obhs, orh, orih, orith, outbuf,

tbdlhs, tbhs, and tribuff, will be treated as black boxes automatically

by the tool.

3. Some hard macroes in Actel design, including dfpc, dfpca, dl2a, dl2b,

dl2c, dl2d, are not supported by this tool and will be treated as black

boxes automatically.

4. A list of black boxes can be found in file "black.box" under the running

directory after retargetter is run.

5. Compiled VML (CVM) distribution files are now supported.

- The executable "cvm2sml" is now shipped with ViewRetargeter. This

executable will be used on vendor distributed libraries, which

are distributed as "CVM" files.  CVM files are "Compiled VML"

files.  A CVM2SML.DOC file is provided with this distribution.

Bug fixes:

1. Load buffering was improved.

2. Various core-dump and infinite loop errors have been fixed.

3. Various SML library data were improved.

Application notes:

1. On retargetting from Xilinx to others, the global hidden signal should

be appropriately programmed. By default, they are connected to VDD.

In the case that global signal is not used at all, it needs to be

connected to GND by using "ground_net" command.

2. Pullup and Pulldown cells are not supported. These kinds of cells will

need to be taken out before running Retargeter.

3. Combinational loops are not allowed in the design. In case that there

are combinational loops in the design, these blocks would need to be

either broken or hidden in black boxes.

4. Every input pins and output pins specified in the symbol file must be

used in the schematic. It is not allowed to have an input node which

has no load, or to have an output node which has no drive.

5. The connected pins of a black box must be given PINTYPEs.

Target libraries:

1. SML continues to be distributed by Viewlogic and the

ViewSynthesis development group.  However, the process of

changing over to a more general library distribution and to

Vendor-based library distribution has begun.

2. The following is a list of the current status of all ViewLogic

distributed cell libraries, the required aliases for the

viewdraw.ini, and whether their data has been significantly

updated in this release.

Library         Vendor          Aliases         Updated in 2.2.1

-------         -------         ------------    --------------

act1            ACTEL           actelcells          yes

actelmodels

actelsim

act2            ACTEL           actelcells          yes

actelmodels

actelsim

act3            ACTEL           actelcells          yes

actelmodels

actelsim

ami2un          AMI                -                no

attperf         ATT                -                no

cg21            Fujitsu            -                no

cgaau           Fujitsu            -                no

cmos5           NEC                -                no

cmos6           NEC                -                no

dio             DATA I/O           -                no

lsi100k         LSI             lsilca              no

lsilcapb

lsi10k          LSI             lsi10k              no

scmos           SMOS               -                no

tc140g          Toshiba         tc140g              yes

tc160g          Toshiba         tc160g              yes

tc160gm         Toshiba         tc160g              yes

tc24sc          Toshiba         tc24sc              yes

tc25sc          Toshiba         tc25sc              yes

vgt350          VLSI            vgt350              yes

vsc350          VLSI            vsc350              yes

vsc370          VLSI            vsc370              yes

vgx700          NCR                -                no

vs700           NCR                -                no

x2000           Xilinx          x2000               yes

mx2000

x3000           Xilinx          x3000               yes

mx3000

x4000           Xilinx          x4000               yes

mx4000

----------------------------------------

Also available directly from the ASIC vendor:

msm30s          OKI

at24k5          Atmel

ViewRetargeter2.2 Release 3/5/93

=================================

Bug Fixes:

1. convert name unnecessarily when -vnt option is used.

2. require license for product 412.

Enhancements:

1. The "tech" command now accepts alias:

RETarget > tech [alias:]library_name

the search path is now:

if giving an alias ---> viewdraw.ini directory paths with the the

specified alias.

if no alias given  ---> primary directory.

viewdraw.ini paths.

WDIR path.

2. The following commands also accept alias:

RETarget > out_dir [alias][:]

RETarget > wire [alias:][file_name]

RETarget > report[alias:][file_name]

to check the usage, use

RETarget > help command_name

3. The resulting WIR file will now contain alias:component

descriptions wherever possible.  This means that more than one

technology can be referenced in your viewdraw.ini, so long as

they all have different aliases.  The following is the "tail"

of a typical viewdraw.ini

DIR [p] . (work)

DIR [r] /VdesII/libraries/rel2.2

DIR [r] /import/viewtools/design_kits/lsi100k/lsilca (lsilca)

DIR [r] /import/viewtools/design_kits/lsi100k/lsilcapb (lsilcapb)

DIR [r] /workview/v41/lib/builtin (builtin)

The technology files are all under the /VdesII/libraries/rel2.2

in this example.

4. Black Box timing information is used. It works in this way:

If a black box is used and its name is matched with a cell name

in the technology library. The timing information of the cell

will be propargated through the black box. If the black box name

can not be matched with cells in technology library, then the timing

information is broken at black box as it used to be.

5. A default technology "package" is now set from the library.

This package determines the default wire load table used in

delay calculations.  This can still be over-ridden by the

standard mechanism of setting a package.  See Bug Fix #4 in

VHDLDes 2.1.1, below.

6. Old .lib and old .sml format libraries are no longer supported(

"old_tech" command is NO longer a valid command ).

ViewRetargeter2.1.1 Release on MS-DOS 1/28/93

==================================================

1. To invoke ViewRetargeter2.1.1 on PC, you should type in

vhdldes -retarget [options]

for example, to see what options are available, simply type in

vhdldes -retarget -help

By typing in "vhdldes -retarget" without giving any options, you will be entering

an interactive text interface  and from there you can issue any retargeter

command.

2. It is suggested that for users who are using MS-DOS 5.0 (or higher) version, to

put the following command in the "autoexec.bat" file

doskey retarget=vhdldes -retarget $1 $2 $3 $4 $5 $6 $7 $8 $9

which sets the alias "retarget" to be "vhdldes -retarget ...". From there,

you may type in the following command to invoke ViewRetargeter

retarget [options]

ViewRetargeter2.1.1 Release 1/11/93

===================================

Notes:

1. ViewRetargeter input libraries for Actel and Xilinx( act2, actel, x2000,

x3000, and x4000) are updated  to  add full asic-required functionality.

ViewRetrageter2.1.1 Beta 11/8/92

====================================

Enhancements:

1) User now can manipulate to program the hidden signals as ground, power, or

primary input through the following commands:

--Usage: RETargeter> ground_net [list_of_net_names]

--       define names as global ground (default GND).

--Usage: RETargeter> vdd_net [list_of_net_names]

--       define names as global power (default VDD).

--Usage: RETargeter> add_pin [list_of_net_names]

--       promote names as top-level pins

2) Three different ways of defining black boxes:

1. specify an alias name and whichever cell with the giving alias name will

be treated as a black box. Following is the command to do it

--Usage: RETargeter> hier_alias [list_of_library_alias]

--       Sets the cells with the the same library alias name listed

--       as black box

2. specify a list of black boxes in a file by using the following command

--Usage: RETargeter> hier_cellfile [list_of_cell_files]

--       list of files which saves the hierarchical cell names

Note: each line in the cell file should contain only one cell name.

3. attach "OPTIMIZE=OFF" attribute to the symbol or component. ( was available

in the first beta release)

3) The "FLOATVAL=" attributes for both net and builtin component are handled. The

program also trace down the hierarchy to find "FLOATVAL=" attribute on a

floating net.

4) Black box can have just output ports (no inputs or in-outs).

5) The "technology" command now reads .SML library files.  SML files are

created from VML (Viewlogic Master Library) format files by

VML2SML (available separately).  The old .LIB library files can still

be read by using a new command:

VHDLDes > old_tech [tech_name]

6) The "tech" search path is now:

current directory

viewdraw.ini directory paths with the proper alias for the SML.

The distributor of the SML file must inform the user of the

required alias name.  None of the current SML files

distributed with this tape have such aliases.  So this

access mechanism cannot be used with them.

WDIR path.

7) If signal buffering is required and a non-inverting buffer is

available in the technology library (and is sufficient), it will

be used instead of an inverter pair.

8) If no buffering will acheive the requirements, the best available

Buffering will be used.

9) There is a new, improved GUI.  Especially useful will be the ability to

set timing constraints using sliders and menus.  However, the new

***>    GUI requires that /tmp be writable by the user.

Bug Fixes:

1) No syntax error will occur in the component statements in gnl file.

2) No syntax error will occur when the net name has ":" or "-" in it.

3) Load calculation now includes wire load delay extrapolated from wire

load table for the current package type.  Previous versions were

correct only within the explicit table range.  This version

extrapolates wire load linearly from the last two elements in

the table.  ** YOU MUST still set a package type explicitly to

install the correct wire load table into the computation.

2) "No Pin Delay" error reported for designs which have no logic

between an input and an output is now properly handled.

==============================================================================

Beta release 6/1/92

This file contains some updated information about retargeter 2.0.

The main sections are:

- Licensing

- Viewdraw.ini file

- Library aliasing

- Converting PLDs

- Known bugs

LICENSING

The product code for Retargeter 2.0 is 321.  You must also have the product

codes for each output technology you wish to use.  These product codes must

be placed in your license.wv file and you should obtain the corresponding

magic number from Viewlogic Sales Administration.

VIEWDRAW.INI FILE

Setting up your viewdraw.ini correctly is important!

- The synthesizer does not use library aliases, so the output library

must be specified first.

- Technology libraries, the ones in which you captured your designs, must

be next.

- Retargeter input libraries with the "d" prefix must be specified third.

- Builtins must be specified and the builtin directory can appear anywhere.

- All technology and input libraries must be aliased.

- The input libraries dio and dbuiltin must be specified regardless of what

input technology you are using.

- To retarget TTL, you must specify the dstd input library.

The retargeter input libraries are distributed on all platforms as megafiles.

This means that your viewdraw.ini file must contain lines like

DIR [m] /workview/lib/dstd (dstd)

where the [m] indicates a megafile.  Using [r] or [w] will not work;

using [rm] or [mr] is fine.  The letter "m" must appear in the brackets.

Example viewdraw.ini files

1. Retargeting from TTL to Xilinx 3000 (Workstation)

DIR [r] /workview/lib/x3000          (x3000)    | Synthesis output library

DIR [r] /workview/lib/mx3000         (mx3000)   | Synthesis output library

DIR [r] /workview/lib/74ls           (v74ls)    | Technology library

DIR [m] /workview/lib/dstd           (dstd)     | Retargeter input library

DIR [m] /workview/lib/dbuiltin       (dbuiltin) | Retargeter input library

DIR [m] /workview/lib/dio            (dio)      | Retargeter input library

DIR [r] /workview/lib/builtin        (builtin)  | Builtins

2. Retargeting from TTL to Xilinx 3000 (PC)

DIR [m] c:\workview\lib\x3000        (x3000)    | Synthesis output library

DIR [m] c:\workview\lib\mx3000       (mx3000)   | Synthesis output library

DIR [m] c:\workview\lib\74ls         (v74ls)    | Technology library

DIR [m] c:\workview\lib\dstd         (dstd)     | Retargeter input library

DIR [m] c:\workview\lib\dbuiltin     (dbuiltin) | Retargeter input library

DIR [m] c:\workview\lib\dio          (dio)      | Retargeter input library

DIR [m] c:\workview\lib\builtin      (builtin)  | Builtins

3. Retargeting from Xilinx 4000 to Actel Act-2 (PC)

DIR [m] c:\workview\lib\a1200\cells  (act2cells)     | Synthesis output library

DIR [m] c:\workview\lib\a1200\models (act2models)           | Synthesis output library

DIR [m] c:\workview\lib\x4000        (x4000)    | Technology library

DIR [m] c:\workview\lib\mx4000       (mx4000)   | Technology library

DIR [m] c:\workview\lib\dx4000       (dx4000)   | Retargeter input library

DIR [m] c:\workview\lib\dbuiltin     (dbuiltin) | Retargeter input library

DIR [m] c:\workview\lib\dio          (dio)      | Retargeter input library

DIR [m] c:\workview\lib\builtin      (builtin)  | Builtins

LIBRARY ALIASING

To retarget your design successfully, you must use the Workview 4.1 library

aliasing scheme.  This is required to resolve naming conflicts between

gates with the same name in different libraries.  For example, the Actel

two-input AND gate is named AND2 and has pins named A, B, Y.  The Viewlogic

two-input AND gate is also named AND2, but has pins named A, B and Q.

To add aliases to an existing design such as a Workview 4.0 design, you should

use the tool "altran", which adds or modifies aliases.  Simply set up your

viewdraw.ini file as specified above; this tells Workview what aliases to

use.  Then type the command "altran -p <project name>" where <project name>

is the name of the project you wish to retarget.  After you have performed

this step, you should view your design again to make sure that aliases

have been added correctly.

CONVERTING PLDS

The present version of the retargeter correctly retargets only those PLDs

for which ViewPLD "schematic models" are available.  If you have created a

PLD using ViewPLD, you can use the "Model Device => Schematic Model" menu

option in ViewPLD to create the schematic model.  Make sure that the symbol

instantiated on your schematic is of type COMPOSITE.

If you have created a PLD using the older Viewlogic PLD tools Makepld and

Jedxlat, you must carry out several steps for each PLD.

1. Run "updatjed <symbolname> <filename>" from the command line.  For

example, if you used the symbol named am22v10 and had a JEDEC file named

binbcd, you would run "updatjed am22v10 binbcd".  This creates a copy of

<symbolname> with the name <filename> so that a custom schematic can be

placed underneath it.

2. Execute the "Model Device => Schematic Model" menu option in ViewPLD.

This creates a schematic of DIO primitives to go underneath the symbol.

3. In your top level schematic, execute the Viewdraw command "Change

Component" from <symbolname> to <filename>.

When retargeter is released for shipment, it will automatically perform

the translation steps for both ViewPLD PLDs and Makepld PLDs.

KNOWN BUGS

Problem: When you are using sequential parts from two different libraries, and

two parts have the same name but different aliases, the program should use

the alias to choose the proper model, but it does not.

Workaround: None; only affects designer who mixes, for example, Toshiba

gates and LSI gates on the same schematic.

Problem: Message reporting wrong number of pins does not give hierarchical

pathname.

Workaround: Use the errorlevel=0 option, and scan through the resulting output

(which prints the hierarchical name for every instance) to find the instance

for which the error occurs.