The Toshiba Firmware Upgrade for CDROM Drives

File name to download:	TOSH-UP.ZIP

Location:	BBS +49 2131 158123, Compuserve Go TOSHGER (soon)

Product Scope:	XM-3501B, XM-3601B, XM-3701B

Sometimes it  becomes necessary to upgrade a CDROM drive with the latest firmware. This can happen if a new CD format or technology has been developed, e.g. CD plus, CD-enhanced format. Therefore Toshiba provide you with a special program, which downloads the firmware into the drive’s flash EPROM.

The download program TFER.EXE is a utility that allows XM-3501B, XM-3601B and XM-3701B Flash EPROM Firmware to be updated via the SCSI BUS using  Adaptec SCSI Host Adapters that have an ASPI software interface.

Contents of this data:

- AUTOEXEC BAT	see instrutions

- CONFIG   SYS	see instructions

- HA50706  HEX	Firmware XM-3501B

- JA50707  HEX 	Firmware XM-3601B

- NA60123  HEX 	Firmware XM-3701B

- TFER     EXE	Download program

What you need:

- Adaptec SCSI Controller, e.g. AHA-1540/1542, AHA-2940, AHA-2920

- Adaptec ASPI-Driver, to be found in the EZ-SCSI Software
- e.g. ASPI4DOS.SYS, ASPI8DOS.SYS or MCAM18XX.SYS
- due to copyright reasons we could not supply Adaptec’s ASPI-Manager

- The CD-ROM  device driver is not needed; it will be ignored if installed

- The download utility  TFER.EXE  from Toshiba

- Your XM-3501B, XM-3601B or XM-3701B CD-ROM drive to be upgraded

The following combinations have been tested and worked fine.

AHA-1540/42	+ ASPI4DOS.SYS 	+  XM-3501B, XM-3601B, XM-3701B

AHA-2940    	+ ASPI8DOS.SYS 	+  XM-3501B, XM-3601B, XM-3701B

AHA-2920	+ MCAM18XX.SYS 	+  XM-3501B, XM-3601B, XM-3701B

TMC-1610	+ MCAM18XX.SYS	+  XM-3701B

HandyCard	+ BTASPI.SYS		+  XM-3501B, XM-3601B, XM-3701B

The quick method to upgrade:

- The ASPI interface must be loaded

- At the DOS prompt type:  TFER [FirmWare.HEX] [FlashROM type]
FirmWare.HEX contains the firmware, FlashROM can be AMD, INTEL. Usually AMD.

- For Example XM-3701B:   TFER NA60123.HEX AMD

The convenient method to upgrade:

The data you downloaded consists of the following files:

- Make a DOS bootable diskette, e.g. FORMAT A:/S

- Copy the downloaded data onto the diskette

- Copy the ASPI-Manager of your Adaptec Controller onto the diskette

- Boot using this new floppy disk

- Put in a standard ISO9660 CD formatted media in the drive

- Choose your combination of controller and CDROM drive from the menu

- Follow the instructions of the TFER.EXE program

Firmware Download Software

for Toshiba CDROM Drives

use Adaptec AHA-2920/40, AHA-1540/42

or Future Domain TMC-1610 (XM-3701B only)

Versions:

XM-3501B   HA50706

XM-3601B   JA50707

XM-3701B   NA60123

Date: 28-Feb.1996

- Boot with this disk