HIGH-SPEED EPROM PROGRAMMER MOD-EMEP-1/4/8

README.DOC

**********

1. MOD-EMEP-X has pull-down menu driver software into 2 diskettes,

First diskette (#1) consists of the following files:

EPP8M4.EXE  : MOD-EMEP-4 modules' driver file.

EPP8M1.EXE  : MOD-EMEP-1 modules' driver file.

README.DOC  : This document.

Second diskette (#2) consists of the following files:

DUMP.EXE    : Dump file to Console in BINARY format.

EDBIN.EXE   : Edit a binary file.

EPP8M8.EXE  : MOD-EMEP-8 modules' driver file.

FILEDIV.EXE : Divide a file into couple files

FILSHUFF.EXE: Assemble couple files into one.

HEXBIN.EXE  : HEX in BINARY file converter.

HEXBIN2.EXE : EXTENDED HEX to BINARY converter.

SEP2MB.EXE  : Seperate a BINARY File large than 2Mbits

SPLIT2.EXE  : 2-way file splitter.

SPILT4.EXE  : 4-way file splitter.

SHUFF2.EXE  : 2-way file shuffler.

SHUFF4.EXE  : 4-way file ahuffler.

2. Revision of EPP8MX.EXE ( X : 1,4,8 )

vision V8.00 : JUN , 17 , 1993

EPP8MX.EXE V8.00 has the following modifications:

a. Software V8.00 can be used with either SAC-101A, System Adaptor Card

or LINK-P1, Printer Port Adaptor Module to control MOD-EMEP-X

hardware.

The software automatically detects the kind of interface used.

b. V8.00 and later versions simplify Read/Program of high-capacity EPROM.

When user selects EPROM size larger than software allocated memory

buffer, software will require user to input disk driver number. After

input, software automatically opens a file (BUFF.TMP) with same EPROM

size to use file buffer when it access EPROM's data.

Example:

Assumption: software allocated memory buffer space is 256K bytes

When programming 27C040(4M bits = 512 Kbytes), allocated memory buffer

becomes insufficient. Software automatically opens a 512 Kbytes file

( Driver:BUFF.TMP ). If file opening is successful,

"File > Driver:BUFF.TMP" will appear below "BUFF SIZE :" inside

the status field.

Upon exiting the main program, Driver:BUFF.TMP is recorded in

EPP8MX.DAT for future use. As not to occupy disk space,

Driver:BUFF.TMP is deleted.

NOTE: For executing the above, take note of the following:

a. If the PC's memory RAM is sufficient, it is suggested that the

excess memory RAM be set into RAM DISK (refer to DOS manual).

This way, the maximum speed can be attained during Read/Program

EPROM and avoid hard disk Read/Write frequency.

b. If the PC has insufficient RAM, it is suggested that the biggest free

space of hard disk of faster access time be used as not to affect the

speed of Read/Program EPROM.

c. If the disk space is insufficient or Read/Write is not possible,

"Warning * Memory/File buffer not enough * Press any key to

continune.." would appear on the screen. If disk buffer is

insufficient, solve the above problem on the disk before reentering

main program.

<<< IMPORTANT AMEND !!! >>>

version V8.15 : April 30, 1994

LINK-P1 HARDWARE REVISION

Due to the drivers of different PC's printer port, We were forced

to make some  changes to the hardware of the LINK-P1, We feel sorry

to ask you to also do the necessary changes  to your  LINK-P1 system.

For those  of you who are using software version V8.10 and prior

version,   the following  procedures is necessary,

1. Open LINK-P1 and take out U2 (Lattice GAL16V8 or AMD PALCE16V8).

2. Use the JEDEC file -- "LINK-P1.JED" to program to the U2.

3. After the programming step is completed ok, put the U2 back.

4. Make sure all the steps stated above is correctly done, continue

to use version V8.15 or later software.